BaiOMC Restful API Definition

（v1.0）

Contents

Part1: RESTful API for configuring, querying and maintenance	4

1.	Create Restful API User	4

2.	Access API	4

2.1.	Token (Baisc request)	4

3.	API Parameters	5

3.1.	Get Device Running Status	5

3.2.	Get Device Parameters	7

3.3.	Get Parameters from Device	10

3.4.	Configure Parameters	11

3.5.	Get IMSI List of HaloB Attached	13

3.6.	Delete IMSI of eNB	16

3.7.	Modify Device Cell Name	17

3.8.	Speed Test	18

4.	Maintenance API	21

4.1.	Reboot	21

4.2.	Reset Factory	22

5.	Jobs API	23

5.1.	Get Job Result API	23

5.2.	Get Jobs List API	25

6.	eNB/CPE Status Report	27

6.1.	Online or offline of eNB Report	27

6.2.	Installed Status of eNB Report	28

6.3.	Online or Offline of CPE	29

6.4.	Infos of CPE	30

7.	Devices and Device Groups	34

7.1.	Get Device Groups	34

7.2.	Add Group	36

7.3.	Modify Group	36

7.4.	Delete Group	37

7.5.	Add Secondary Group	38

7.6.	Modify Secondary Group	39

7.7.	Delete Secondary Group	40

7.8.	Get Devices	41

7.9.	Add Device	43

7.10.	Modify Device	44

7.11.	Delete Device	45

8.	IPsec&CA Certificate API	46

8.1.	Upload CA Certificate	46

8.2.	Upload Ipsec Certificate	47

8.3.	GET IPSec Certificate List	48

8.4.	GET CA Certificate List	51

8.5.	Issue Certificate to Device	54

8.6.	Get Issued Certificate Status	55

9.	Status Code Description	57

Part2: LDAP	58

1. What is LDAP	58

1.1. LDAP Functions	58

1.2. LDAP Basic Model	58

1.3. LDAP Architecture	59

2. Client Configuration	59

LDAP Server Base DN (Base Distinguished Name)	60

3. Login OMC	60

3.1. Login	60

3.2. User and Group	61

Part1: RESTful API for configuring, querying and maintenance

- Create Restful API User

Login the OMC with admin account, select “System > Settings” to create users for northbound restful interfaces.

- Access API

Generate an access token with the username and password.

Token (Baisc request)

HTTP protocol, POST method

Content-Type:application/json;charset=UTF-8

URL:http://ip:port/northboundApi/v1/access/token

Request

{

"username": ,

"password":

}

| Parameters | Type | Description |
| --- | --- | --- |
| username | string(50) | Username to access via api |
| password | string(50) | Password to access via api |

Response:

{

"code": 0,

"data": {

"token": "dasfdfdf1dsdffdv",

"expires": 1800

},

"message": ""

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Responded data |
| message | string | Responded message |
| token | string | token |
| expires | Int | Expired of token(s) |

- API Parameters

Get Device Running Status

HTTP protocol, GET method.

URL:/v1/enodeb/infos/status/{sn}

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| sn | string(50) | Serial number of the eNB |

Response:

{

"code": 0,

"data": {

"cellStatus": "",

"lockStatus": "",

"mmeStatus": "",

"connectionStatus": "",

"rfStatus": "",

"sn": "",

"syncStatus": ""

},

"message": ""

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | string(256) | Returned data |
| cellStatus | string | Cell State 0:Not Activated; 1:Activated |
| lockStatus | string | Lock status 0:unlock; 1:locked |
| mmeStatus | string | Status of MME |
| connectionStatus | string | Online status off: offline; on: online |
| rfStatus | string | Radio Frequency Switch 0:Off; 1:On |
| sn | string | eNB id |
| syncStatus | string | Synchronization status 0:failed; 1:succeed |
| message | string | Returned information |

Example

Request

curl --location --request GET 'http://192.168.14.27:8081/northboundApi/v1/enodeb/infos/status/1202000091177SP0060' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2NDk5MjE3MzMsInVzZXJuYW1lIjoiYWRtaW4ifQ.P8ADnQVK50-Dz4ZsBo6QCvlpwFyZps8qo-YiGJubzyc'

Response

{ "code": 0, "data": { "cellStatus": "", "lockStatus": "", "mmeStatus": "", "connectionStatus": "", "rfStatus": "", "sn": "", "syncStatus": "" }, "message": "" }

Get Device Parameters

HTTP protocol, GET method.

URL: http://ip:port/northboundApi/v1/device/parameters/{sn}

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| sn | string(50) | Serial number of the eNB |

Response:

{

"code": 200,

"message": "success",

"data": {

"hostName": null,

"gpsLongitude": "117.0950",

"halobFlag": "0",

"firstOnlineTime": "2022-06-21T06:03:02.000+0000",

"eafcn": "210",

"opState": 0,

"eci": "232323",

"mmePool1": "",

"networkModel": "FDDMode",

"mmePool2": "",

"mac": "48:BF:74:01:4D:69",

"productName": "V4-SC",

"mmeStatusPool": "",

"pci": null,

"tac": "3",

"cellIp": "20.88.0.31",

"onlineTime": "2022-06-22T02:27:24.000+0000",

"offlineTime": "2022-06-22T02:26:34.000+0000",

"mmeStatus": "{}",

"firmwareVersion": "B01",

"mmepoolIpsecAddr": "20.88.0.31,null,null",

"s1siglinkserverlist": "10.0.0.150",

"gpsLatitude": "31.8328",

"gpsVersion": "--",

"product": "QAFA",

"serialNumber": "1202000089177LP0008",

"moduleType": "pBS4212",

"bandwidth": null,

"gpsHeight": "229.0000",

"newMmeStatus": null,

"maxtxpower": "0",

"upTime": "0d 0h 17m 52s",

"rfStatus": 1,

"smallCellCode": "48BF74_1202000089177LP0008",

"connectionStatus": "on",

"siteId": null,

"mmeEnable": 0,

"plmnid": "46068",

"softwareVersion": "BaiBS_QAFA_2.2.1"

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |
| hostName | string | eNB name |
| cellIp | string | eNB IP address |
| rfStatus | Int | Radio Frequency Switch 0:Off; 1:On |
| opState | Int | Cell State 0:Not Activated; 1:Activated |
| ECI | string | CELL_IDENTITY |
| PCI | string | PHYCELLID |
| mmeStatus | string | Status of MME |
| mac | string | Mac Address |
| product | string | Product |
| productName | string | Product name |
| softwareVersion | string | Software Version |
| firstOnlineTime | string | Time of first online |
| onlineTime | string | Online time |
| offlineTime | string | Offline time |
| connectionStatus | string | Online status off: offline on: online |
| plmnid | string | PLMN ID |
| gpsHeight | string | Height of the GPS |
| gpsLatitude | string | Latitude of the GPS |
| gpsLongitude | string | Longitude of the GPS |
| halobFlag | Int | HaloB status 0:Close; 1: On |
| serialNumber | string | eNB ID |
| smallCellCode | string | eNB code |
| eafcn | string | frequency |
| bandwidth | string | bandwidth |
| tac | string | Trace Area Code |
| mmepoolIpsecAddr | string | IPSec address |
| networkModel | string | Network model |
| maxtxpower | int | Max transmit power of the eNB |
| s1siglinkserverlist | string | MME IP |
| mmePool1 | string | MME Pool1 IP |
| mmePool2 | string | MME Pool2 IP |
| upTime | string | Running time of the system |

Example

Request

curl http://192.168.14.27:8081/northboundApi/v1/device/parameters/1202000091177SP0060 -X GET -K -H “Authorization:1lIjoiYWRtaW4ifQ.pHSjw93Nq5o9ht1ic2akEPg-ed40sbJ9ifjIwVtAfTA”

Response

{ "code": 200, "message": "success", "data": { "hostName": null, "gpsLongitude": "117.0950", "halobFlag": "0", "firstOnlineTime": "2022-06-21T06:03:02.000+0000", "eafcn": "210", "opState": 0, "eci": "232323", "mmePool1": "", "networkModel": "FDDMode", "mmePool2": "", "mac": "48:BF:74:01:4D:69", "productName": "V4-SC", "mmeStatusPool": "", "pci": null, "tac": "3", "cellIp": "20.88.0.31", "onlineTime": "2022-06-22T02:27:24.000+0000", "offlineTime": "2022-06-22T02:26:34.000+0000", "mmeStatus": "{}", "firmwareVersion": "B01", "mmepoolIpsecAddr": "20.88.0.31,null,null", "s1siglinkserverlist": "10.0.0.150", "gpsLatitude": "31.8328", "gpsVersion": "--", "product": "QAFA", "serialNumber": "1202000089177LP0008", "moduleType": "pBS4212", "bandwidth": null, "gpsHeight": "229.0000", "newMmeStatus": null, "maxtxpower": "0", "upTime": "0d 0h 17m 52s", "rfStatus": 1, "smallCellCode": "48BF74_1202000089177LP0008", "connectionStatus": "on", "siteId": null, "mmeEnable": 0, "plmnid": "46068", "softwareVersion": "BaiBS_QAFA_2.2.1" } }

Get Parameters from Device

HTTP protocol, GET method.

URL: http://ip:port/northboundApi/v1/device/parameters/query/{sn}

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter:

["Device.DeviceInfo.SoftwareVersion","Device.Services.FAPService.1.AccessMgmt.LTE.HNBName","Device.DeviceInfo.SAS.FAP.GPS.LockedLatitude","Device.DeviceInfo.SAS.FAP.GPS.LockedLongitude"]

| Parameters | Type | Description |
| --- | --- | --- |
| path | string(256) | Parameter path |
| sn | string(50) | Serial number |

Response:

{

"code": 200,

"message": "",

"data": {

"sn": "120200044821BSB0020",

"jobId": "af30d7371d6a477bb73d44e4f3300997",

"waitTime": 5

}

}

Example

Request

curl --location --request POST 'http://172.21.175.129:8081/northboundApi/v1/device/parameters/query/120200044821BSB0020' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE3MDE0MDA2NzgsInVzZXJuYW1lIjoid3lxIn0.zbwG-NJs-8ZEe97fI0lCfIgbyMnu33j_S69LXzw3_1U' \ --header 'Content-Type: application/json' \ --data-raw ' ["Device.DeviceInfo.SoftwareVersion","Device.Services.FAPService.1.AccessMgmt.LTE.HNBName","Device.DeviceInfo.SAS.FAP.GPS.LockedLatitude","Device.DeviceInfo.SAS.FAP.GPS.LockedLongitude"]'

Response

{ "code": 200, "message": "", "data": { "sn": "120200044821BSB0020", "jobId": "af30d7371d6a477bb73d44e4f3300997", "waitTime": 5 } }

Configure Parameters

HTTP protocol, PUT method.

Content-Type:application/json;charset=UTF-8

URL: http://ip:port/northboundApi/v1/device/parameters/{sn}

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter:

{

"pathList": [

{

"path": "Device.Services.FAPService.1.FAPControl.LTE.HSS.HaloBEnableState",

"value": true

}

]

}

| Parameters | Type | Description |
| --- | --- | --- |
| path | string(256) | Parameter path |
| value | string(256) | Parameter value |
| sn | string(50) | Serial number |

Response:

{

"code": 200,

"data": {

"jobId": "1",

"waitTime": "5"

},

"message": "success"

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |
| jobId | string | Task ID |
| waitTime | int | Wait time for result queries. The unit is second. |

Example

Request

curl --location --request PUT 'http://192.168.14.27:8081/northboundApi/v1/device/parameters/1202000089177LP0008' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2NTcwMTU0MzEsInVzZXJuYW1lIjoiYWRtaW4ifQ.SNUtqxFvFXsGQYeHKfvmQscqu9f0ggqMAcBzeeAw7Cw' \ --header 'Content-Type: application/json' \ --data-raw '{ "pathList": [ { "path": "Device.Services.FAPService.1.AccessMgmt.LTE.HNBName", "value": "beijing" } ] }'

Response

{ "code": 200, "data": { "jobId": "1", "waitTime": "5" }, "message": "success" }

Standard path refers to "North bound interface standard parameters.xlsx":

Get IMSI List of HaloB Attached

HTTP protocol, GET method.

URL: http://ip:port/northboundApi/v1/enodeb/infos/imsi/{sn}

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter:

| Parameters | Type | Description |
| --- | --- | --- |
| sn | string(50) | Serial number |
| startTime | string(50) | Start time |
| endTime | string(50) | End time |

Response:

{

"code": 200,

"message": "success",

"data": {

"call": [

{

"imsi": "0001",

"smallCellCode": "48BF74_1202000091177SP0005",

"status": null,

"failureReason": null,

"requestTime": "2022-03-16T11:30:38",

"responseTime": null,

"apnStatus": null,

"apnRequestTime": null,

"apnResponseTime": null

}

],

"traffic": [

{

"imsi": "001",

"smallCellCode": "48BF74_1202000091177SP0005",

"status": "1",

"failureReason": null,

"requestTime": "2022-03-16T11:33:38",

"responseTime": "2022-03-16T11:33:40",

"apnStatus": null,

"apnRequestTime": "2022-03-16T11:33:43",

"apnResponseTime": "2022-03-16T11:33:44"

}

]

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| message | string(256) | Returned information |
| data | object | Returned data |
| call | array | Call |
| imsi | string | IMSI |
| smallCellCode | string | Small cell code |
| status | string | 0: deleted; 1: allocated; 2: allocation failed |
| failureReason | string | Failure reason |
| requestTime | Date | Small cell request time |
| responseTime | Date | OMC response time |
| apnStatus | string | 0: allocation failed; 1: allocation succeeded |
| apnRequestTime | Date | Small cell request APN time |
| apnResponseTime | Date | OMC response APN time |
| traffic | array | Traffic |

Example

Request

curl --location --request GET 'http://192.168.14.27:8081/northboundApi/v1/enodeb/infos/imsi/1202000091177SP0060?endTime=2022-04-08%2016:00:00&startTime=2022-04-07%2000:00:00' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2NDk5MjE3MzMsInVzZXJuYW1lIjoiYWRtaW4ifQ.P8ADnQVK50-Dz4ZsBo6QCvlpwFyZps8qo-YiGJubzyc'

Response

{ "code": 200, "message": "success", "data": { "call": [ { "imsi": "0001", "smallCellCode": "48BF74_1202000091177SP0005", "status": null, "failureReason": null, "requestTime": "2022-03-16T11:30:38", "responseTime": null, "apnStatus": null, "apnRequestTime": null, "apnResponseTime": null } ], "traffic": [ { "imsi": "001", "smallCellCode": "48BF74_1202000091177SP0005", "status": "1", "failureReason": null, "requestTime": "2022-03-16T11:33:38", "responseTime": "2022-03-16T11:33:40", "apnStatus": null, "apnRequestTime": "2022-03-16T11:33:43", "apnResponseTime": "2022-03-16T11:33:44" } ] } }

Delete IMSI of eNB

HTTP protocol, DELETE method.

Content-Type:application/json;charset=UTF-8

URL:http://ip:port/northboundApi/v1/enodeb/infos/enodeb/imsi

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| smallCellCode | string(50) | Cell code, oui serial number |
| cmdType | string | clear |
| imsi | string |  |

Param:

{

"cmdType":"clear",

"smallCellCode":"48BF74_1202000240194DP0026",

"imsi":"200010001012333"

}

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"message": ""

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | string(256) | Returned information |
| message | string | Returned data |

Example

Request

curl --location --request DELETE 'http://172.21.175.129:8081/northboundApi/v1/enodeb/infos/enodeb/imsi' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2NjUzMDQ0NDIsInVzZXJuYW1lIjoidXNlcjEyMzQ1NDUifQ.UbBnkoG-06uPIQh8vidwBZnh7DwSD_C1ZjCiSepHwNM' \ --header 'Content-Type: application/json' \ --data-raw '{ "cmdType":"clear", "smallCellCode":"48BF74_1202000240194DP0026", "imsi":"200010001012333" }'

Response

{ "code": 200, "message": "success", "data": null }

Modify Device Cell Name

HTTP protocol, PUT method.

URL: http://ip:port/northboundApi/v1/device/parameters/cellname/{sn}

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter:

| Parameters | Type | Description |
| --- | --- | --- |
| sn | string(50) | Serial number |
| cellName | string(50) | Cell name |
| syncFlag | Int | 0 |

Parameter:

{

"cellName": "baicells002",

"syncFlag":0

}

Response:

{

"code": 200,

"message": "success",

"data": null

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| message | string(256) | Returned information |
| data | object | Returned data |

Example

Request

curl --location --request PUT 'http://172.21.175.129:8081/northboundApi/v1/device/parameters/cellname/12150000092183B0096' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2NjE0MTQwNjgsInVzZXJuYW1lIjoiYWRtaW4ifQ.UVIx_QYH9DrpwiCShgjZ1xdLGfvzyZaVOSCEHDKvN-c' \ --header 'Content-Type: application/json' \ --data-raw '{ "cellName": "baicells002", "syncFlag":0 }

Response

{ "code": 200, "message": "success", "data": null }

Speed Test

HTTP protocol, POST method.

URL: http://ip:port/northboundApi/v1/enodeb/infos/{sn}/speed

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter:

| Parameters | Type | Description |
| --- | --- | --- |
| sn | string(50) | Serial number |
| serialNumber | string(50) | serialNumber |
| speedTestHost | string | speedTestHost Portland, OR-Comcast;Madison, WI-Hoyos Consulting LLC;Portland, OR-Comcast;Phoenix, AZ-Sparklight;Buenos Aires-Metrotel;London-Orbital Net;Oslo-Telia Norge AS speedtest.get.no:8080 |

Parameter:

{

"serialNumber":"1202000291216HB0003",

"speedTestHost":"speedtest.get.no:8080"

}

Response:

{

"code": 200,

"message": "",

"data": {

"sn": "1202000291216HB0003",

"jobId": "c137d45af8f34606a66be90704667d4f",

"waitTime": 5

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |
| jobId | string | Task ID |
| waitTime | int | Wait time for result queries unit: seconds |

Example

Request

curl --location --request POST 'http://172.21.175.129:8081/northboundApi/v1/enodeb/infos/1202000291216HB0003/speed' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE3MDU5OTM4MjksInVzZXJuYW1lIjoid3lxIn0.vUmsTdlhmOiqeHBBC7JgZZd_sWinDM0vJLsqoe9XJOk' \ --header 'Content-Type: application/json' \ --data-raw '{ "serialNumber":"1202000291216HB0003", "speedTestHost":"speedtest.get.no:8080" }'

Response

{ "code": 200, "message": "", "data": { "sn": "1202000291216HB0003", "jobId": "c137d45af8f34606a66be90704667d4f", "waitTime": 5 } }

TIP：Refer to Interface 4.1 for the returned jobId

Request

curl --location --request GET 'http://172.21.175.129:8081/northboundApi/v1/job/result/c137d45af8f34606a66be90704667d4f' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE3MDU5OTM4MjksInVzZXJuYW1lIjoid3lxIn0.vUmsTdlhmOiqeHBBC7JgZZd_sWinDM0vJLsqoe9XJOk' '

Response

{ "code": 200, "message": "success", "data": { "jobId": "c137d45af8f34606a66be90704667d4f", "name": "Query ennoeb speed", "status": "2", "sn": null, "errorMessage": null, "createTime": "2024-01-23T07:30:39", "completeTime": "2024-01-23T07:30:47", "details": "{\"result\":{\"downloadSpeed\":\"4.85,17.46,9.79,8.93,9.99,9.51,9.99,9.70\",\"uploadSpeed\":\"10.60,5.83,5.51,5.20,5.56,5.25,5.61,5.30\",\"delayTime\":\"449\"},\"success\":true,\"uuid\":\"c137d45af8f34606a66be90704667d4f\"}" } }

- Maintenance API

Reboot

HTTP protocol, POST method.

URL: http://ip:port/northboundApi/v1/device/reboot/{sn}

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| sn | string(50) | Serial number |

Response:

{

"code": 200,

"data": {

"jobId": "1",

"waitTime": "240"

},

"message": "success"

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |
| jobId | string | Task ID |
| waitTime | int | Wait time for result queries. The unit is second. |

Example

Request

curl --location --request POST 'http://192.168.14.27:8081/northboundApi/v1/device/reboot/1202000089177LP0008' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2NTcwMTM4OTYsInVzZXJuYW1lIjoiYWRtaW4ifQ.NfxZpRN8PmvQjyk7yiPlTVzXFMgVta5cxTD_kuP0h3U' \ --data-raw ''

Response

{ "code": 200, "data": { "jobId": "1", "waitTime": "240" }, "message": "success" }

Reset Factory

HTTP protocol, POST method.

URL: http://ip:port/northboundApi/v1/device/reset/{sn}

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter:

| Parameters | Type | Description |
| --- | --- | --- |
| sn | string(50) | Serial number |

Response:

"code": 200,

"data": {

"jobId": "1",

"waitTime": "300"

},

"message": "success"

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |
| jobId | string | Task ID |
| waitTime | int | Wait time for result queries. The unit is second. |

Example

Request

curl --location --request POST 'http://192.168.14.27:8081/northboundApi/v1/device/reset/1202000089177LP0008' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2NTcwMTM4OTYsInVzZXJuYW1lIjoiYWRtaW4ifQ.NfxZpRN8PmvQjyk7yiPlTVzXFMgVta5cxTD_kuP0h3U' \ --data-raw ''

Response

{ "code": 200, "data": { "jobId": "1", "waitTime": "240" }, "message": "success" }

- Jobs API

Get Job Result API

HTTP protocol, GET method.

URL: http://ip:port/northboundApi/v1/job/result/{jobId}

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter:

| Parameters | Type | Description |
| --- | --- | --- |
| jobId | string(32) | Task ID |

Response:

{

"code": 200,

"message": "success",

"data": {

"jobId": "af30d7371d6a477bb73d44e4f3300997",

"name": "get param value",

"status": "2",

"sn": "120200044821BSB0020",

"errorMessage": null,

"createTime": "2023-12-01T03:20:52",

"completeTime": "2023-12-01T03:20:55",

"details": "{\"serialNumber\":\"120200044821BSB0020\",\"success\":true,\"parameterList\":[{\"path\":\"Device.DeviceInfo.SoftwareVersion\",\"value\":\"BaiBLQ_3.2.1\"},{\"path\":\"Device.DeviceInfo.SAS.FAP.GPS.LockedLatitude\",\"value\":\"30538787\"},{\"path\":\"Device.DeviceInfo.SAS.FAP.GPS.LockedLongitude\",\"value\":\"104066986\"},{\"path\":\"Device.Services.FAPService.1.AccessMgmt.LTE.HNBName\",\"value\":\"Nova-436Q-120200044821BSB0020\"}],\"uuid\":\"af30d7371d6a477bb73d44e4f3300997\"}"

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |
| jobId | string | Task ID |
| status | Int | 0:In progress; 1:Success; 2:Success; 3:Failure |
| errorMessage | string | Failed error message |
| createTime | string | Create time |
| completeTime | string | Complete time |
| details | string | JSON String result of “3.3 Get Parameters from Device” |

Example

Request

curl --location --request GET 'http://192.168.14.27:8081/northboundApi/v1/job/result/eb2ba6bdc9bd400d8a936ffc540df8e4' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2NTcwMTU0MzEsInVzZXJuYW1lIjoiYWRtaW4ifQ.SNUtqxFvFXsGQYeHKfvmQscqu9f0ggqMAcBzeeAw7Cw'

Response

{ "code": 200, "message": "success", "data": { "jobId": "af30d7371d6a477bb73d44e4f3300997", "name": "get param value", "status": "2", "sn": "120200044821BSB0020", "errorMessage": null, "createTime": "2023-12-01T03:20:52", "completeTime": "2023-12-01T03:20:55", "details": "{\"serialNumber\":\"120200044821BSB0020\",\"success\":true,\"parameterList\":[{\"path\":\"Device.DeviceInfo.SoftwareVersion\",\"value\":\"BaiBLQ_3.2.1\"},{\"path\":\"Device.DeviceInfo.SAS.FAP.GPS.LockedLatitude\",\"value\":\"30538787\"},{\"path\":\"Device.DeviceInfo.SAS.FAP.GPS.LockedLongitude\",\"value\":\"104066986\"},{\"path\":\"Device.Services.FAPService.1.AccessMgmt.LTE.HNBName\",\"value\":\"Nova-436Q-120200044821BSB0020\"}],\"uuid\":\"af30d7371d6a477bb73d44e4f3300997\"}" } }

Get Jobs List API

HTTP protocol, POST method.

Content-Type:application/json;charset=UTF-8

URL:http://ip:port/northboundApi/v1/job/result/page

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter:

{

"status": "",

"searchText": "",

"startTime":"",

"endTime":"",

"page":1,

"rows":10,

"timeZone":0

}

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| status | int | 0:In progress; 1:Success; 2:Success; 3:Failure |
| searchText | string(32) | Fuzzy query |
| startTime | string(32) | startTime(yyyy-MM-dd HH:mm:ss) |
| endTime | string(32) | endTime(yyyy-MM-dd HH:mm:ss) |
| page | int | page |
| rows | int | rows |
| timeZone | int | Time Zone |

Response:

{

"code": 200,

"message": "success",

"data": {

"pageNo": 1,

"pageSize": 50,

"totalPage": 1,

"totalRows": 1,

"rows": [

{

"jobId": "f73c05d15eb947e8bc8a65da4fe90550",

"name": "Modify config param value",

"status": "2",

"sn": "12150000092183B0096",

"errorMessage": null,

"createTime": "2022-08-25T06:17:48",

"completeTime": "2022-08-25T06:17:50"

}

]

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |
| jobId | string | Task ID |
| status | Int | 0:In progress 1:Success 2:Success 3:Failure |
| errorMessage | string | Failed error message |
| createTime | string | Create time |
| completeTime | string | Complete time |
| pageNo | Int | pageNo |
| pageSize | Int | pageSize |
| totalPage | Int | totalPage |
| totalRows | Int | totalRows |
| rows | Array | rows |

Example

Request

curl --location --request GET 'http://172.17.9.86:8081/northboundApi/v1/job/result/page \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2NTcwMTU0MzEsInVzZXJuYW1lIjoiYWRtaW4ifQ.SNUtqxFvFXsGQYeHKfvmQscqu9f0ggqMAcBzeeAw7Cw'

Response

{ "code": 200, "message": "success", "data": { "pageNo": 1, "pageSize": 50, "totalPage": 1, "totalRows": 1, "rows": [ { "jobId": "f73c05d15eb947e8bc8a65da4fe90550", "name": "Modify config param value", "status": "2", "sn": "12150000092183B0096", "errorMessage": null, "createTime": "2022-08-25T06:17:48", "completeTime": "2022-08-25T06:17:50" } ] } }

- eNB/CPE Status Report

Online or offline of eNB Report

HTTP protocol, POST method.

Content-Type:application/json;charset=UTF-8

URL:

Parameter:

| Parameters | Type | Description |
| --- | --- | --- |
| sn | string(32) | Serial number of the eNB |
| connected | Boolean | Whether connected, true/false |
| recordTime | string(32) | yyyy-MM-dd HH:mm:ss) |

Parameter:

[

{

"sn": "12020004803675H",

"connected": true,

"recordTime": "2022-09-29 21:42:45"

}

]

Response:

{

"code": 200

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |

Example

Request

curl --location --request POST 'http://localhost:8081/api/xxxx' \ --header 'Content-Type: application/json' \ --data-raw '[ { "sn": "12020004803675H", "connected": true, "recordTime": "2022-09-29 21:42:45" } ]'

Response

{ "code": 200 }

Installed Status of eNB Report

HTTP protocol, POST method.

Content-Type:application/json;charset=UTF-8

URL:

Parameter:

| Parameters | Type | Description |
| --- | --- | --- |
| sn | string(32) | Serial number of the eNB |
| online | string | 1:on; 0:off |
| active | string | 1:active; 0:inactive |

Parameter:

{

"sn": "1202000480225VH0221",

"online": "1"

}

O

{

"sn": "1202000480225VH0221",

"active": "1"

}

Response:

{

"resultCode": 200,

"code": 200,

"resultMsg": null,

"message": null,

"tid": 1663056308400,

"data": true

}

| Parameters | Type | Description |
| --- | --- | --- |
| resultCode | int | Status code |
| code | int | Status code |
| data | boolean | True/false |
| resultMsg | String | Result message |
| message | String | message |
| tid | String | tid |

Online or Offline of CPE

HTTP protocol, GET method.

Content-Type:application/json;charset=UTF-8

URL: http://${HOST_IP}:8081/v1/cpe/status/${sn}

Parameter:

| Parameters | Type | Description |
| --- | --- | --- |
| serialNumber | string(32) | Serial number of the CPE |

Required parameter:

${sn}: serial number of the device

Response parameters:

| Parameters | Type |  | Description |  |
| --- | --- | --- | --- | --- |
| code | int |  | Status code |  |
| message | string(32) |  | Additional information |  |
| connectionStatus |  | string(32) |  | Connected status, on/off |
| serialNumber | string(32) |  | Serial number |  |

Example

Request

curl -XGET 'http://${HOST_IP}:8081/v1/cpe/status/120300008019ABP0909' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2NTcwMTU0MzEsInVzZXJuYW1lIjoiYWRtaW4ifQ.SNUtqxFvFXsGQYeHKfvmQscqu9f0ggqMAcBzeeAw7Cw'

Response

{ "code": 200, "message": "success", "data": { "serialNumber": "120300008019ABP0909", "connectionStatus": "off" } }

Infos of CPE

HTTP protocol, GET method.

Content-Type:application/json;charset=UTF-8

URL: http://${HOST_IP}:8081/v1/cpe/infos/${sn}

Parameter:

| Parameters | Type | Description |
| --- | --- | --- |
| serialNumber | string(32) | Serial number of the CPE |

Required parameter:

${sn}: serial number of the device

Response parameters:

| Parameters | Type |  | Description |  |
| --- | --- | --- | --- | --- |
| code | int |  | Status code |  |
| message | string(32) |  | Additional information |  |
| connectionStatus |  | string(32) |  | Connection status, on or off |
| serialNumber | string(32) |  | Serial number |  |
| bandWidth | string(32) |  | Band width |  |
| lgwIp | string(32) |  | LGW IP address |  |
| nrRsrp | string(32) |  | 5g RSRP |  |
| dlEarfcn | string(32) |  | EARFCN of downlink |  |
| nrBand | string(32) |  | 5g frequency band |  |
| firstOnlineTime | string(32) |  | The initial online time. |  |
| nrRsrq | string(32) |  | 5g RSRP |  |
| cpeSinr | string(32) |  | SINR of CPE |  |
| mcc | string(32) |  | mcc |  |
| nrBandwidth | string(32) |  | 5g bandwidth |  |
| nrEarfcn | string(32) |  | 5g EARFCN |  |
| nrCinr | string(32) |  | 5g CINR |  |
| cpeName | string(32) |  | CPE name |  |
| macaddress | string(32) |  | Mac address |  |
| ulCurrentDataRate | string(32) |  | Uplink data rate |  |
| longitude | string(32) |  | longitude |  |
| nrPlmn | string(32) |  | 5g PLMN |  |
| dlFrequency | string(32) |  | Downlink frequency |  |
| ulMcs | string(32) |  | Uplink MCS |  |
| dlBler | string(32) |  | Downlink BLER |  |
| rsrp0 | string(32) |  | Rsrp0 |  |
| rsrp1 | string(32) |  | Rsrp1 |  |
| nrPci | string(32) |  | 5g PCI |  |
| nrUlFrequency | string(32) |  | 5g uplink frequency |  |
| nrDlFrequency | string(32) |  | 5g downlink frequency |  |
| lastinformtime | string(32) |  | The last heartbeat time |  |
| softwareVersion | string(32) |  | Software version |  |
| lteStatus | string(32) |  | LTE status |  |
| ipaddress | string(32) |  | IP address |  |
| ulFrequency | string(32) |  | Uplink frequency |  |
| dlMcs | string(32) |  | Downlink LCS |  |
| cellName | string(32) |  | eNB name |  |
| latitude | string(32) |  | latitude |  |
| lteConnectionTime | string(32) |  | LTE connection time |  |
| cellIdentity | string(32) |  | Cell ID |  |
| imsi | string(32) |  | IMSI |  |
| txPower | string(32) |  | Transmit power |  |
| moduleVersion | string(32) |  | Module version |  |
| pci | string(32) |  | PCI |  |
| onlineTime | string(32) |  | Online time |  |
| offlineTime | string(32) |  | Offline time |  |
| nrSinr | string(32) |  | 5g SINR |  |
| nrCellId | string(32) |  | 5g cell id |  |
| cinr0 | string(32) |  | Cinr0 |  |
| product | string(32) |  | Reported product name |  |
| cinr1 | string(32) |  | Cinr1 |  |
| mnc | string(32) |  | MNC |  |
| uptime | string(32) |  | System running time |  |
| dlCurrentDataRate | string(32) |  | Downlink throughput |  |
| modelName | string(32) |  | Module name |  |
| marketName | string(32) |  | Market name |  |
| linkCondition | string(32) |  | Linked Condition |  |
| lastsyntime | string(32) |  | The last synchronizing time |  |

Example

Request

curl -XGET 'http://localhost:8081/v1/cpe/infos/120300008019ABP0909' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2NTcwMTU0MzEsInVzZXJuYW1lIjoiYWRtaW4ifQ.SNUtqxFvFXsGQYeHKfvmQscqu9f0ggqMAcBzeeAw7Cw'

Response

{ "code": 200, "message": "success", "data": { "ulMcs": "2.0", "dlMcs": "19.0", "rsrp0": "-74.3", "rsrp1": "-69.7", "cinr0": "15.8", "cinr1": "18.3", "pci": "36.0", "dlCurrentDataRate": "0.0", "ulCurrentDataRate": "0.0", "uptime": "14d 2h 12m 0s", "offLineTime": "2023-11-28 06:18:04", "onLineTime": "2023-11-01 08:11:02", "imsi": "460680005000069", "macAddress": "48:BF:74:0C:8E:BF", "ipAddress": "192.168.101.179", "cellName": "", "ulFrequency": "3640000", "cpeCode": "48BF740C8EBF", "serialNumber": "120300008019ABP0909", "cellIdentity": "4399036", "cpeName": "120300008019ABP0909", "softwareVersion": "BaiCE_BM_2.5.26_NA", "connectionStatus": "Off", "lastInformTime": "2023-11-02 05:19:09", "lastSynTime": "2023-11-02 05:19:09", "cpeSinr": "9.4", "modelName": "EG7035E-M11", "lgwIp": "172.21.172.14:50069", "lgwMac": "", "txPower": "-4.9", "mnc": "68", "mcc": "460.0", "lteStatus": "connected", "bandwidth": "10", "groupName": "Default Level Group/Default Device Group", "longitude": "", "latitude": "", "height": "", "firstOnlineTime": "2023-07-29 10:24:18", "linkCondition": "", "dlEarfcn": "43990", "cpeModel": "4G", "dlBler": "", "marketName": "Atom OD04H-M11", "nrPci": "", "nrBand": "", "nrBandwidth": "", "nrEarfcn": "", "nrPlmn": "", "nrCellId": "", "nrDlFrequency": "", "nrUlFrequency": "", "nrCinr": "", "nrSinr": "", "nrRsrp": "", "nrRsrq": "", "moduleVersion": "4.2.2.0-30436-BYPASS-1.1.3", "lteConnectionTime": "0d 0h 6m 54s", "imei": "865436030488506", "lgwUrl": "https://172.21.172.14:50069", "distance": "", "moduleName": "MLH3B11" } }

- Devices and Device Groups

Get Device Groups

HTTP protocol, GET method.

URL:http://ip:port/northboundApi/v1/device/group

Content-Type:application/json;charset=UTF-8

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| searchText | string(50) | Search text |

Parameter：

{

"searchText":

}

Response:

{

"code": 200,

"message": "success",

"data": {

"total": 3,

"rows": [

{

"id": "1",

"children": [

{

"id": 30,

"group_name": "Default Device Group",

"description": null,

"built_in": 1

},

{

"id": 112,

"group_name": "12345",

"description": "123",

"built_in": 0

},

{

"id": 113,

"group_name": "111",

"description": "2",

"built_in": 0

},

{

"id": 114,

"group_name": "test",

"description": "test",

"built_in": 0

}

],

"group_name": "Default Level Group",

"built_in": 1

},

{

"id": "2",

"children": [],

"group_name": "1234",

"built_in": 0

},

{

"id": "3",

"children": [],

"group_name": "test123update",

"built_in": 0

}

],

"footer": [],

"properties": null

}

}

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| id | Int | ID |
| group_name | String(200) | Group name |
| description | String(200) | Description |
| built_in | Int | Built-in device group 1-Y; 0-N |
| children | Array | children |

Add Group

HTTP protocol, POST method.

Content-Type:application/json;charset=UTF-8

URL:http://ip:port/northboundApi/v1/device/group

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| groupName | string(200) | Group name |

Parameter：

{

"groupName":"test123"

}

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"groupId": "3"

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| groupId | Int | Group ID |

Modify Group

HTTP protocol, PUT method.

Content-Type:application/json;charset=UTF-8

URL:http://ip:port/northboundApi/v1/device/group

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| groupId | Int | Group ID |
| groupName | string(200) | Group name |

Parameter：

{

"groupId":3,

"groupName":"test123update"

}

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"message": ""

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |

Delete Group

HTTP protocol DELETE method

URL:http://ip:port/northboundApi/v1/device/group

Content-Type:application/json;charset=UTF-8

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| groupId | Int | Group ID |

Parameter：

{

"groupId":3

}

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"message": ""

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |

Add Secondary Group

HTTP protocol POST method

URL:http://ip:port/northboundApi/v1/device/group/sub

Content-Type:application/json;charset=UTF-8

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| name | string(200) | Group Name, required |
| desc | string(200) | Group description |
| ids | string(200) | Ids |
| pid | int | Parent Group ID, required |

Remark：

For obtaining Ids, refer to “7.8 Get Devices” for obtaining the Device list interface

Id=small_cell_code+_+product

Parameter:

{

"name":"test123",

"desc":"test123",

"ids":"48BF74_1202000089177LP0008_FAP/pBS4212/SC,48BF74_12020003952147B0031_FAP/pBS3101S/DC",

"pid":"1"

}

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"message": "Add successfully."

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |

Modify Secondary Group

HTTP protocol PUT method

URL:http://ip:port/northboundApi/v1/device/group/sub

Content-Type:application/json;charset=UTF-8

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| name | string(200) | Group name, required |
| desc | string(200) | Group description |
| ids | string(200) | Ids |
| groupId | int | Group ID, required |

Remark：

For obtaining Ids, refer to “7.8 Get Devices” for obtaining the Device list interface

Id=small_cell_code+_+product

Parameter：

{

"name":"test",

"desc":"test",

"ids":"48BF74_1215000009214CS0011_FAP/pCRB2000/DC,48BF74_12020004212185B0016_FAP/pCRB2000/SC",

"groupId":"117"

}

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"message": ""

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |

Delete Secondary Group

HTTP protocol DELETE method

URL:http://ip:port/northboundApi/v1/device/group/sub

Content-Type:application/json;charset=UTF-8

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| groupId | Int | Group ID |

Parameter：

{

"groupId":3

}

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"message": ""

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |

Get Devices

HTTP protocol POST method

URL:http://ip:port/northboundApi/v1/device/query

Content-Type:application/json;charset=UTF-8

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| isGnb | Int | 0 |
| groupId | Int | Group ID |
| searchText | String | Search text |
| pageSize | Int | Page size |
| pageNo | Int | Page number |

Parameter：

{

"isGnb":"0" ,

"groupId":30 ,

"searchText":null,

"pageSize":10,

"pageNo":0

}

Response:

{

"code": 200,

"message": "success",

"data": {

"total": 1,

"rows": [

{

"small_cell_code": "48BF74_1202000089177LP0008",

"serial_number": "1202000089177LP0008",

"mac_address": "48:BF:74:01:4D:69",

"host_name": "unknown name",

"software_version": "BaiBS_QAFA_2.2.1",

"dual_carrier_type": 0,

"group_name": "Default Level Group/Default Device Group",

"product": "FAP/pBS4212/SC",

"connection_status": "On",

"longitude": "117.0950",

"latitude": "31.8329",

"height": "222.0000",

"manufacturer": "BAICELLS",

"module_type": "pBS4212",

"maxtxpower": "-20",

"cell_ip": "** unavailable **\n",

"tac": "1"

}

],

"footer": [],

"properties": null

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| small_cell_code | String(200) | small_cell_code |
| serial_number | String(200) | serial_number |
| host_name | String(200) | eNB name |
| group_name | Int | group_name |
| product | String(200) | Product |
| connection_status | String | Online status off: offline; on: online |
| longitude | double | longitude |
| latitude | double | latitude |
| height | double | height |
| cell_ip | string | eNB IP address |
| mac_address | string | Mac address |
| software_version | string | Software version |
| manufacturer | string | manufacturer |
| dual_carrier_type | string | DC/CA |
| module_type | string | Module type |
| maxtxpower | string | Max transmit power |
| tac | Int | Trace area code |

Add Device

HTTP protocol POST method

URL:http://ip:port/northboundApi/v1/device

Content-Type:application/json;charset=UTF-8

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| groupId | string(200) | Group name |
| userName | string(50) | admin |
| serialNumber | String(500) | Join with ; |
| isGnb |  | 0 |

Parameter：

{

"userName":"admin",

"groupId":"30",

"serialNumber":"sn00000001;sn00000002",

"isGnb":"0"

}

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"message": "Add device successfully"

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |

Modify Device

HTTP protocol PUT method

URL:http://ip:port/northboundApi/v1/device

Content-Type:application/json;charset=UTF-8

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| cellCode | String(50) | Base station serial number |
| groupName | string(200) | Group name |
| longitude | Double | longitude |
| latitude | Double | latitude |
| height | Double | height |

Parameter：

{

"cellCode":"48BF74_12150000092183B0089",

"longitude":15.12,

"latitude":16.62,

"height":30

}

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"message": ""

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |

Delete Device

HTTP protocol DELETE method

URL:http://ip:port/northboundApi/v1/device

Content-Type:application/json;charset=UTF-8

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameters:

| Parameters | Type | Description |
| --- | --- | --- |
| ids | Int | Join with , |
| userName | string(50) | admin |

Remark：

Id=small_cell_code+_+product

Refer to “7.8 Get Devices”

Parameter：

{

"ids":"48BF74_12150000092183B0089_FAP/pCRB2000/SC,48BF74_12150000092183B0088_FAP/pCRB2000/SC",

"userName":"admin"

}

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"message": ""

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | object | Returned data value |
| message | string | Returned information |

- IPsec&CA Certificate API

Upload CA Certificate

HTTP protocol, POST method.

Content-Type:multipart/form-data

URL:http://ip:port/northboundApi/v1/enodeb/infos/cert/uploadIpsecCACertFile

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| uploadFile | File | CA Certificate file, required |
| nbType | String | Default is ‘eNB’, required |
| description | String | Description |

Parameter：

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"message": ""

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | string(256) | Returned data |
| success | Bool | Whether success True; false |
| message | string | Lock status 0:unlock; 1:locked |
| message | string | Returned information |

Example

Request

curl --location --request POST 'http://172.21.175.129:8081/northboundApi/v1/enodeb/infos/cert/uploadIpsecCACertFile' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2OTkwMDAyMDEsInVzZXJuYW1lIjoid3lxIn0.XfJq4hCGpE2f-Z9eCFz29WaUr1QlzpQ4A4WHPHe7osE' \ --form 'uploadFile=@"/C:/Users/baicells/Downloads/120200044821BSB0020_20231101-031101_cert/20231101-031650_caCert.der"' \ --form 'nbType="eNB"' \ --form 'description="123467"'

Response

{ "code": 200, "message": "success", "data": { "success": true, "message": "" } }

Upload Ipsec Certificate

HTTP protocol, POST method.

Content-Type:multipart/form-data

URL:http://ip:port/northboundApi/v1/enodeb/infos/cert/uploadIpsecPrivateCertFile

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| uploadCertFile | File | IPSec Certificate file, required |
| uploadSecretKeyFile | File | IPSec key file, required |
| serialNumber | String | SN, required |
| nbType | String | Default is ‘eNB’, required |
| description | String | Description |

Parameter：

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"message": ""

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | string(256) | Returned data |
| success | Bool | true false |
| message | string | Lock status 0:unlock; 1:locked |
| message | string | Returned information |

Example

Request

curl --location --request POST 'http://172.21.175.129:8081/northboundApi/v1/enodeb/infos/cert/uploadIpsecPrivateCertFile' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2OTkwMDAyMDEsInVzZXJuYW1lIjoid3lxIn0.XfJq4hCGpE2f-Z9eCFz29WaUr1QlzpQ4A4WHPHe7osE' \ --form 'uploadCertFile=@"/C:/Users/baicells/Downloads/120200044821BSB0020_20231101-031101_cert/ENBCert.der"' \ --form 'uploadSecretKeyFile=@"/C:/Users/baicells/Downloads/120200044821BSB0020_20231101-031101_cert/ENBKey.der"' \ --form 'nbType="eNB"' \ --form 'description="1234"' \ --form 'serialNumber="120200044821BSB0020"'

Response

{ "code": 200, "message": "success", "data": { "success": true, "message": "" } }

GET IPSec Certificate List

HTTP protocol, POST method.

Content-Type:application/json

URL:http://ip:port/northboundApi/v1/enodeb/infos/cert/manager/list

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| page | int | Page pagination functionality |
| rows | int | Rows pagination functionality |

Parameter：

{

"page":0,

"rows":10

}

Response:

{

"code": 200,

"message": "success",

"data": {

"pageNo": 1,

"pageSize": 10,

"totalPage": 1,

"totalRows": 1,

"rows": [

{

"caFileName": "20231101-031650_caCert.der",

"caUploadStatus": "1",

"serialNumber": "120200044821BSB0020",

"certUploadStatus": "3",

"secretKeyUploadStatus": "3",

"uploader": "north(wyq)",

"description": "1234",

"secretKeyFileName": "ENBKey.der",

"id": 9,

"uploadTime": "2023-11-03 08:34:23",

"certFileName": "ENBCert.der"

}

]

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | string(256) | Returned data |
| pageNo | int | Page number |
| pageSize | int | Page size |
| totalPage | int | Total page |
| totalRows | int | Total rows |
| rows | Array |  |
| caFileName | String | CA Certificate |
| caUploadStatus | int | CA Issue status 0- Failed, 1- succeeded, 2- In progress, 3-unissued |
| serialNumber | String | Serial number |
| certFileName | String | IPSec file name |
| certUploadStatus | int | IPSec Issue Status 0- Failed, 1- succeeded, 2- In progress, 3-unissued |
| secretKeyFileName | String | IPSec key name |
| secretKeyUploadStatus | int | IPSec key Issue Status 0- Failed, 1- succeeded, 2- In progress, 3-unissued |
| id | int | ID |
| uploader | String | uploader |
| uploadTime | String | Upload time |

Example

Request

curl --location --request POST 'http://172.21.175.129:8081/northboundApi/v1/enodeb/infos/cert/manager/list' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2OTkwMDAyMDEsInVzZXJuYW1lIjoid3lxIn0.XfJq4hCGpE2f-Z9eCFz29WaUr1QlzpQ4A4WHPHe7osE' \ --header 'Content-Type: application/json' \ --data-raw '{ "page":0, "rows":10 }'

Response

{ "code": 200, "message": "success", "data": { "pageNo": 1, "pageSize": 10, "totalPage": 1, "totalRows": 1, "rows": [ { "caFileName": "20231101-031650_caCert.der", "caUploadStatus": "1", "serialNumber": "120200044821BSB0020", "certUploadStatus": "3", "secretKeyUploadStatus": "3", "uploader": "north(wyq)", "description": "1234", "secretKeyFileName": "ENBKey.der", "id": 9, "uploadTime": "2023-11-03 08:34:23", "certFileName": "ENBCert.der" } ] } }

GET CA Certificate List

HTTP protocol, POST method.

Content-Type:application/json

URL:http://ip:port/northboundApi/v1/enodeb/infos/cert/CA/list

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| page | int | Page pagination functionality |
| rows | int | Rows pagination functionality |

Parameter：

{

"page":0,

"rows":10

}

Response:

{

"code": 200,

"message": "success",

"data": {

"pageNo": 1,

"pageSize": 10,

"totalPage": 1,

"totalRows": 2,

"rows": [

{

"fileName": "20231101-031650_caCert.der",

"uploader": "north(wyq)",

"description": "123467",

"id": 7,

"uploadTime": "2023-11-03 08:30:12"

},

{

"fileName": "20231101-031650_caCert.der",

"uploader": "north(wyq)",

"description": "123467",

"id": 6,

"uploadTime": "2023-11-02 09:06:17"

}

]

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | string(256) | Returned data |
| pageNo | int | Page serial number |
| pageSize | int | Page size |
| totalPage | int | Total pages |
| totalRows | int | Total rows |
| rows | Array |  |
| fileName | String | CA Certificate file name |
| id | int | ID |
| uploader | String | uploader |
| uploadTime | String | Upload time |
| description | String | description |

Example

Request

curl --location --request POST 'http://172.21.175.129:8081/northboundApi/v1/enodeb/infos/cert/CA/list' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2OTkwMDAyMDEsInVzZXJuYW1lIjoid3lxIn0.XfJq4hCGpE2f-Z9eCFz29WaUr1QlzpQ4A4WHPHe7osE' \ --header 'Content-Type: application/json' \ --data-raw '{ "page":0, "rows":10 }'

Response

{ "code": 200, "message": "success", "data": { "pageNo": 1, "pageSize": 10, "totalPage": 1, "totalRows": 2, "rows": [ { "fileName": "20231101-031650_caCert.der", "uploader": "north(wyq)", "description": "123467", "id": 7, "uploadTime": "2023-11-03 08:30:12" }, { "fileName": "20231101-031650_caCert.der", "uploader": "north(wyq)", "description": "123467", "id": 6, "uploadTime": "2023-11-02 09:06:17" } ] } }

Issue Certificate to Device

HTTP protocol, POST method.

Content-Type:application/json

URL:http://ip:port/northboundApi/v1/enodeb/infos/cert/send

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| id | Int | Refer id from “8.3 GET IPSec Certificate List” |
| caCertEnable | Int | Whether to issue the CA certificate 1:true; 0:false |
| caCertId | int | Refer id from “8.4 GET CA Certificate List“ When “caCertEnable” is set to “1”, required |
| deviceCertEnable | int | Whether to issue Iec&Key certificate 1:true; 0:false |
| nbType | String | Default is eNB |

Parameter：

{

"id": 5,

"caCertEnable": 1,

"deviceCertEnable": 1,

"caCertId": 5,

"nbType": "eNB"

}

Response:

{

"code": 200,

"message": "success",

"data": {

"success": true,

"message": ""

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | string(256) | Returned data |
| success | Bool | true false |
| message | string | Lock status 0:unlock; 1:locked |
| message | string | Returned information |

Example

Request

curl --location --request POST 'http://172.21.175.129:8081/northboundApi/v1/enodeb/infos/cert/send' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2OTkwMDAyMDEsInVzZXJuYW1lIjoid3lxIn0.XfJq4hCGpE2f-Z9eCFz29WaUr1QlzpQ4A4WHPHe7osE' \ --header 'Content-Type: application/json' \ --data-raw '{ "id": 5, "caCertEnable": 1, "deviceCertEnable": 1, "caCertId": 5, "nbType": "eNB" }'

Response

{ "code": 200, "message": "success", "data": { "success": true, "message": "" } }

Get Issued Certificate Status

HTTP protocol, GET method.

Content-Type:application/json

URL:http://ip:port/northboundApi/v1/enodeb/infos/cert/status/{sn}

Message header

| Parameters | Type | Description |
| --- | --- | --- |
| Authorization | string(50) | token |

Parameter

| Parameters | Type | Description |
| --- | --- | --- |
| sn | sn | Serial number, required |

Parameter：

Response:

{

"code": 200,

"message": "success",

"data": {

"caFileName": "20231101-031650_caCert.der",

"caUploadStatus": "1",

"serialNumber": "120200044821BSB0020",

"certUploadStatus": "3",

"secretKeyUploadStatus": "3",

"uploader": "north(wyq)",

"description": "1234",

"secretKeyFileName": "ENBKey.der",

"updateTime": null,

"id": 9,

"uploadTime": "2023-11-03 08:34:23",

"certFileName": "ENBCert.der"

}

}

| Parameters | Type | Description |
| --- | --- | --- |
| code | int | Status code |
| data | string(256) | Returned data |
| caFileName | String | CA certificate name |
| caUploadStatus | int | CA Issue Status 0- Failed, 1- succeeded, 2- In progress, 3-unissued |
| serialNumber | String | Serial number |
| certFileName | String | IPSec file name |
| certUploadStatus | int | IPSec Issue Status 0- Failed, 1- succeeded, 2- In progress, 3-unissued |
| secretKeyFileName | String | IPSec key name |
| secretKeyUploadStatus | int | IPSec key Issue Status 0- Failed, 1- succeeded, 2- In progress, 3-unissued |
| id | int | id |
| uploader | String | uploader |
| uploadTime | String | Upload time |

Example

Request

curl --location --request GET 'http://172.21.175.129:8081/northboundApi/v1/enodeb/infos/cert/status/120200044821BSB0020' \ --header 'Authorization: eyJ0eXAiOiJKV1QiLCJhbGciOiJIUzI1NiJ9.eyJpYXQiOjE2OTkwMDAyMDEsInVzZXJuYW1lIjoid3lxIn0.XfJq4hCGpE2f-Z9eCFz29WaUr1QlzpQ4A4WHPHe7osE'

Response

{ "code": 200, "message": "success", "data": { "caFileName": "20231101-031650_caCert.der", "caUploadStatus": "1", "serialNumber": "120200044821BSB0020", "certUploadStatus": "3", "secretKeyUploadStatus": "3", "uploader": "north(wyq)", "description": "1234", "secretKeyFileName": "ENBKey.der", "updateTime": null, "id": 9, "uploadTime": "2023-11-03 08:34:23", "certFileName": "ENBCert.der" } }

- Status Code Description

| Status Code | Description |
| --- | --- |
| 200 | Request succeeded |
| 401 | Request user authentication |
| 500 | Server internal error, unable to complete request |
| 502 | An invalid response was received from the remote server when the server working as a gateway or proxy attempted to execute the request |

Part2: LDAP

- What is LDAP

Lightweight Directory Access Protocol (LDAP) is an open, neutral, industry-standard application protocol that provides access control and maintenance of directory information for distributed information through an IP protocol.

  - LDAP Functions

- Users’ account are stored in a centralized manner

- Single Sign-On（SSO）

  - LDAP Basic Model

| Attribute | Name | Description |
| --- | --- | --- |
| c | country | A two-digit country code. For example, CN, US, HK, JP, etc. |
| cn | common name | Usually refers to the name of an object. If you are a person, use your full name. For example, CN=ldapadmin |
| dc | domain component | Usually a domain name is specified. Example: DC=lenovo,DC=com |
| dn | distinguished name | Unique identifier, similar to an absolute path, each object has a unique identifier. example: uid=tester,ou=People,dc=example,dc=com |
| o | organization |  |
| ou | organization unit | Usually specify the name of an organizational unit. Example: OU= Industry 8.0,DC=hspv,DC=com |
| rdn | relative distinguished name | Relative identification name, similar to relative path. Example: uid=tester |
| sn | sur name | Usually refers to a person's surname. Example: sn: Wang |
| uid | user id | Usually refers to the user login name. Example: uid=tester |

  - LDAP Architecture

- A user enters the username and password to try to login the OMC system.

- The OMC performs the login authentication internally (DB). If the authentication succeeds, then user login succeeds.

- If the OMC cannot find the stored account in DB, the OMC authenticates the login account from the LDAP service.

- Client Configuration

- Login the OMC with admin account, select “System->Settings->LDAP Protocol” page.

| Name | Explanation | Remark |
| --- | --- | --- |
| LDAP Enable | Enable or disable LDAP protocol |  |
| LDAP IP | LDAP Server IP |  |
| LDAP Port | LDAP Server Port |  |
| LDAP Base | - LDAP Server Base DN (Base Distinguished Name) |  |
| LDAP User | LDAP Server Login username |  |
| LDAP Password | LDAP Server Login password |  |

- After the configuration completed, click Test to verify whether the LDAP server is available.

Normal UnNormal

- Login OMC

  - Login

| Name | Explanation | Remark |
| --- | --- | --- |
| userName | Username that the user exists in the LDAP server |  |
| password | password that the user exists in the LDAP server |  |

  - User and Group

If a user login in the OMC with LDAP server user, the OMC will record the user account into the OMC DB at the same time. By doing this, OMC can find the account in OMC DB immediately if this user login next time. The User Group of this user would be stored as Default Group, and it can be modified by super administrator.